2006 -08 Multifamily Energy Efficiency Rebate Program (MFEER) Workpaper

Item:  Occupancy Sensors (Wall or Ceiling)

Application:  Interior Residential or Common Area Lighting

Issue:

2005 DEER does not provide savings information for residential multifamily occupancy sensors.

Assumptions:

- No previous sensor installed

- Controls 388 watts of T12 (roughly 8 T12 lamps and corresponding ballasts)

- Occupancy sensor generates 20% savings based on the CEC’s 2005 CEC Impact Analysis.

- 3.33 hours of operation per day. Reference CFL Metering Final Report, Table 4-2 (p. 4-2).

Gross Savings Calculation (kWh):

((385w/1000) x 20%savings) x (3.3hrs x 365days) = 93.5 kWh

Gross Capacity Reductions (kW):

0.089 kW (SDG&E Statewide Estimate)

Effective Useful Life Calculation:

Apply DEER values for occupancy sensors (e.g. DEER CALC00AVOCC01)

Gross IMC:

SDG&E Statewide Estimate

Net-To-Gross

Apply Residential Contractor Program ratio

Table 1

| Measure | Gross Capacity (kW) | Source | Gross Energy Savings (kWh) |
| --- | --- | --- | --- |
| T-8 or T-5 Lamp and Electronic, 2-foot lamp installed 1st lamp | 0.010 | SW Express Team | 82 |
| T-8 or T-5 Lamp and Electronic, 2-foot lamp installed 2nd lamp | 0.010 | SW Express Team | 82 |
| T-8 or T-5 Lamp and Electronic, 2-foot lamp installed 3rd lamp | 0.010 | SW Express Team | 82 |
| T-8 or T-5 Lamp and Electronic, 2-foot lamp installed 4th lamp | 0.010 | SW Express Team | 82 |
| T-8 or T-5 Lamp and Electronic, 2-foot lamp Removed | 0.038 | SW Express Team | 312 |
| T-8 or T-5 Lamp and Electronic, 3-foot lamp installed 1st lamp | 0.012 | SW Express Team | 97 |
| T-8 or T-5 Lamp and Electronic, 3-foot lamp installed 2nd lamp | 0.012 | SW Express Team | 97 |
| T-8 or T-5 Lamp and Electronic, 3-foot lamp installed 3rd lamp | 0.012 | SW Express Team | 97 |
| T-8 or T-5 Lamp and Electronic, 3-foot lamp installed 4th lamp | 0.012 | SW Express Team | 97 |
| T-8 or T-5 Lamp and Electronic, 3-foot lamp removed | 0.044 | SW Express Team | 366 |
| T-8 or T-5 Lamp and Electronic, 4-foot lamp installed 1st lamp | 0.011 | SW Express Team | 89 |
| T-8 or T-5 Lamp and Electronic, 4-foot lamp installed 2nd lamp | 0.011 | SW Express Team | 89 |
| T-8 or T-5 Lamp and Electronic, 4-foot lamp installed 3rd lamp | 0.011 | SW Express Team | 89 |
| T-8 or T-5 Lamp and Electronic, 4-foot lamp installed 4th lamp | 0.011 | SW Express Team | 89 |
| T-8 or T-5 Lamp and Electronic, 4-foot lamp removed | 0.043 | SW Express Team | 358 |
| T-8 or T-5 Lamp and Electronic, 8-foot lamp installed 1st lamp | 0.009 | SW Express Team | 75 |
| T-8 or T-5 Lamp and Electronic, 8-foot lamp installed 2nd lamp | 0.009 | SW Express Team | 75 |
| T-8 or T-5 Lamp and Electronic, 8-foot lamp installed 3rd lamp | 0.009 | SW Express Team | 75 |
| T-8 or T-5 Lamp and Electronic, 8-foot lamp installed 4th lamp | 0.009 | SW Express Team | 75 |
| T-8 or T-5 Lamp and Electronic, 8-foot lamp removed | 0.079 | SW Express Team | 657 |