2006 -08 Multifamily Energy Efficiency Rebate Program (MFEER) Workpaper

Item:  Compact Fluorescent Fixture – 18 watt

Application:  Exterior lighting (dusk-to-dawn)

Issue:

2005 DEER does not address the multifamily exterior compact fluorescent fixture installation replacing an incandescent lamp.

Assumptions:

- Circuit is controlled by timer or dusk-to-dawn sensor

- Replaces:  Incandescent fixture equal to 75 watts

- Annual hours:  4,458

Sources:

- Run-time Hours:  Southern California Edison’s “Work Paper Supporting the Residential Lighting Incentive Program Inputs for Energy Savings, Load Impacts, and Cost-Effectiveness” date June 1, 2006.

kWh Savings Calculation:

(75watts - 18watts) * 4458 hours / 1,000 = 254.11 kWh

Gross Coincident Peak kW

None.

Effective Useful Life Calculation:

Apply DEER values for interior compact fluorescent fixtures (e.g. DEER RRes00AVM18St)

Gross IMC:

Apply DEER values for interior compact fluorescent fixtures (e.g. DEER RRes00AVM18St)

Net-To-Gross

Apply Residential Contractor Program ratio