2006 -08 Multifamily Energy Efficiency Rebate Program (MFEER) Workpaper

Item:  Screw-In Compact Fluorescent Lamp – 23 watt

Application:  Exterior lighting (dusk-to-dawn)

Issue:

2005 DEER does not address the multifamily exterior CFL installation replacing an incandescent lamp.

Assumptions:

- Circuit is controlled by timer or dusk-to-dawn sensor

- Replaces:  Incandescent – 100 watt

- Annual hours:  4,458

Sources:

- Run-time Hours:  Southern California Edison’s “Work Paper Supporting the Residential Lighting Incentive Program Inputs for Energy Savings, Load Impacts, and Cost-Effectiveness” date June 1, 2006.

kWh Savings Calculation:

(100watts - 23watts) * 4458 hours / 1,000 = 343.27 kWh

Gross Coincident Peak kW

None.

Effective Useful Life Calculation:

Note:  DEER utilizes CFL life rating of 8,000 hours to determine EUL.

8000 / 4458 = 1.8 EUL

Gross IMC:

Apply DEER values for interior screw-in CFLs (e.g., DEER RRes00AVI25St)

Net-To-Gross

Apply Residential Contractor Program ratio