2006 -08 Multifamily Energy Efficiency Rebate Program (MFEER) Workpaper

Item:  Linear Fluorescent T8 or T5 Lamps

Application:  Underground lighting (90%)/Dawn to Dusk Lighting (10%)

Issue:

2005 DEER does not provide savings information for a non-premium T12 conversion to T8.

Assumptions:

- Replaces:  Non-energy saving T12 Lamp

- Annual hours:  8,322:  (7,884 Underground, 438 Dawn to dusk)

Gross Savings Calculations (kWh):

Gross Peak * 8322 hours = kWh

See Table 1 below for calculations

Gross Capacity Reductions (kW)

Reference:  SW Express Efficiency Program (2003)

Effective Useful Life Calculation:

Apply DEER values for linear fluorescent lamps (e.g. DEER CECC00AVT8E01)

Gross IMC:

Reference:  SW Express Efficiency Program (2003)

Net-To-Gross

Apply Residential Contractor Program ratio

Table 1

| Measure | Gross Capacity (kW) | Source | Gross Energy Savings (kWh) |
| --- | --- | --- | --- |
| T-8 or T-5 Lamp and Electronic, 2-foot lamp installed 1st lamp | 0.010 | SW Express Team | 82 |
| T-8 or T-5 Lamp and Electronic, 2-foot lamp installed 2nd lamp | 0.010 | SW Express Team | 82 |
| T-8 or T-5 Lamp and Electronic, 2-foot lamp installed 3rd lamp | 0.010 | SW Express Team | 82 |
| T-8 or T-5 Lamp and Electronic, 2-foot lamp installed 4th lamp | 0.010 | SW Express Team | 82 |
| T-8 or T-5 Lamp and Electronic, 2-foot lamp Removed | 0.038 | SW Express Team | 312 |
| T-8 or T-5 Lamp and Electronic, 3-foot lamp installed 1st lamp | 0.012 | SW Express Team | 97 |
| T-8 or T-5 Lamp and Electronic, 3-foot lamp installed 2nd lamp | 0.012 | SW Express Team | 97 |
| T-8 or T-5 Lamp and Electronic, 3-foot lamp installed 3rd lamp | 0.012 | SW Express Team | 97 |
| T-8 or T-5 Lamp and Electronic, 3-foot lamp installed 4th lamp | 0.012 | SW Express Team | 97 |
| T-8 or T-5 Lamp and Electronic, 3-foot lamp removed | 0.044 | SW Express Team | 366 |
| T-8 or T-5 Lamp and Electronic, 4-foot lamp installed 1st lamp | 0.011 | SW Express Team | 89 |
| T-8 or T-5 Lamp and Electronic, 4-foot lamp installed 2nd lamp | 0.011 | SW Express Team | 89 |
| T-8 or T-5 Lamp and Electronic, 4-foot lamp installed 3rd lamp | 0.011 | SW Express Team | 89 |
| T-8 or T-5 Lamp and Electronic, 4-foot lamp installed 4th lamp | 0.011 | SW Express Team | 89 |
| T-8 or T-5 Lamp and Electronic, 4-foot lamp removed | 0.043 | SW Express Team | 358 |
| T-8 or T-5 Lamp and Electronic, 8-foot lamp installed 1st lamp | 0.009 | SW Express Team | 75 |
| T-8 or T-5 Lamp and Electronic, 8-foot lamp installed 2nd lamp | 0.009 | SW Express Team | 75 |
| T-8 or T-5 Lamp and Electronic, 8-foot lamp installed 3rd lamp | 0.009 | SW Express Team | 75 |
| T-8 or T-5 Lamp and Electronic, 8-foot lamp installed 4th lamp | 0.009 | SW Express Team | 75 |
| T-8 or T-5 Lamp and Electronic, 8-foot lamp removed | 0.079 | SW Express Team | 657 |