By:	SCE Design & Engineering Services

Date:	May 27, 2005

Subject:	Energy Star Room Air Conditioners Annual Energy Savings and Summer Demand Reduction Engineering Estimates

Background

The Residential Energy Efficiency Rebate Program includes incentives for Energy Star qualified Room Air Conditioning units.  In California, the minimum efficiency requirements for Room Air Conditioners are governed by the Title 20 Appliance Standards, which reflect the national appliances standards set by the U.S. Department of Energy.  This document describes how the estimated above-code demand and energy savings for this measure were derived.

Methodology

The estimated annual energy savings and summer demand reduction for Energy Star qualified room air conditioners for each of the California Energy Commission Title 24 climate zones (CZ) within Southern California Edison (SCE) Company’s service area are summarized in Table 1.

| Table 1. Energy Star Room Air Conditioners Savings per Climate Zone. |  |  |  |  |
| --- | --- | --- | --- | --- |
| CZ | DEER Annual Energy Consumption (kWh) per Multi-Family Apartment (RASS Weight Averaged) | Above Code Percent Energy Savings | Above Code Annual Savings (kWh per Unit) | Above Code Demand Reduction (kW per Unit) |
| 6 | 313 | 17.8% | 56 | 0.280 |
| 8 | 620 | 17.8% | 110 | 0.280 |
| 9 | 909 | 17.8% | 161 | 0.280 |
| 10 | 1,109 | 17.8% | 197 | 0.280 |
| 13 | 2,209 | 17.8% | 392 | 0.280 |
| 14 | 1,346 | 17.8% | 239 | 0.280 |
| 15 | 3,626 | 17.8% | 644 | 0.280 |
| 16 | 346 | 17.8% | 61 | 0.280 |

The table values were derived as follows:

- From a scoping study1 for Room AC Recycling performed by RLW for SCE in 2001, the following information was obtained for the analysis:

  - The assumed Room AC unit system size was 14,000 BTU per Hour (1.2 tons);

  - Since the California Title 20 requirements range from an 8.5 EER up to 9.8, the representative efficiency of 8.8 EER in the scoping study was deemed to characterize the above code baseline unit with a demand of 1.59 kW; and

  - An Energy Star qualified Room AC unit was characterized as a 10.7 EER unit with a demand of 1.31 kW.

- The estimated above code demand reduction was taken as the difference between the demand of the 8.8 EER Room AC unit and the demand of the 10.7 EER Room AC unit:

1.59 kW – 1.31 kW = 0.28 kW

- The demand reduction was assumed to be equivalent across all eight climate zones within SCE’s service area.  This assumption was based on considerations that during typical summer weather conditions and a three-day heat wave, from the coastal region to the inland deserts, the Room AC units will operate at or above the ARI EER test condition of 92˚F.  As a result, expected variations in peak demand for same sized Room AC units should be within a few percentage points across the different climate zones during a summer heat wave, with increasing demand towards the desert areas.  This assumption simplifies the demand estimation and builds-in conservatism into the estimate, since the estimated demand based on the ARI EER ratings will under-predict the potential demand reduction for the hotter climates zones, while remaining close to the anticipated demand for the coastal climates during summer peak conditions.

- The Above Code Percent Energy Savings value was determined, again using data from the RLW Room AC Recycling scoping study, as follows:

  - The estimated energy consumption savings from a 7.0 EER Room AC to a baseline 8.8 EER unit is estimated at 37,585 MWh (based on 122,500 units).

  - The estimated energy consumption savings from the 7.0 EER unit to an Energy Star 10.7 EER unit is estimated at 63,540 MWh.

  - Thus, the increase in savings from the above code baseline Room AC unit to an Energy Star Room AC unit, on a per Room AC unit basis, is estimated as:

IncreasedSavings=63,540−37,585122,500⋅1,000=211.8 kWh per Room AC unit

  - The baseline annual energy consumption was estimated as follows:

Baseline Annual Energy Use = 750 hours/year X 1.59 kW = 1,192.5 kWh/year

  - The DEER estimates for total air conditioning annual energy consumption for multi-family building vintages for each climate zone was used as the bases.  These values were then weight averaged to arrive at a single annual energy consumption value for each climate zone.  Attachment 1 describes the data and approach used to derive the annual energy consumption values per climate zone in Table 1.

  - Using information from the 2003 RASS on the year multi-family apartments were built in SCE’s service territory, a building vintage distribution was created that allows the vintage consumptions in each climate zone to be weight averaged into a single representative value for each climate zone (Attachment 1).

  - The closest value to the estimated baseline Room AC annual energy consumption was determined to be CZ 10 with an annual consumption of 1,109 kWh, .i.e., 93% of the baseline estimate.

  - Using the ratio of the CZ 10 energy consumption to the baseline unit energy consumption (93%), an estimate of the savings and percent savings of the annual energy consumption for CZ 10 may be obtained:

Energy Savings for CZ 10 = 211.8 kWh/year X 93% = 197 kWh/year

Percent Savings for CZ 10 = 1971,109X 100 = 17.8%

  - The estimated Percent Energy Savings for CZ 10 was assumed to apply to the remaining climate zones.  Thus, the annual energy savings per climate zone were obtained by multiplying each climate zone DEER weight averaged AC energy consumption by the Percent Savings for CZ 10. The results are summarized in Table 1.

References

1. SCE Residential Room Air-Conditioner Recycling Scoping Study, RLW, 2001.

2. California Residential Appliance Saturation Study, 2003.

3. Engineering Excel Workbook (embedded below).

Attachment 1

DEER Annual Air Conditioning Energy Consumption per Climate Zone

Weight Averaged Using RASS Multi-Family Building Vintage Distributions for Building Vintage and Average Dwelling Sizes

Besides the demand and energy savings estimates, the detailed DEER records for each measure also contain the estimated end use annual energy consumption normalized to the measure’s specified units.  For the purpose of estimating potential air conditioning savings for multi-family applications for measures not yet available in the frozen draft DEER dataset, version 1.0, as of May 2005, SCE’s engineering staff used the total annual air conditioning end use consumption, normalized to dwelling area, from the records for the Multi-Family Direct Evaporative Cooler measures.  The following DEER “RunID” records and values were used:

| RunID | PEBaseE kWh/1,000sqft | RunID | PEBaseE kWh/1,000sqft |
| --- | --- | --- | --- |
| RMFm0603RDEvp | 147.846 | RMFm0803RDEvp | 350.89 |
| RMFm0605RDEvp | 106.762 | RMFm0805RDEvp | 258.096 |
| RMFm0675RDEvp | 388.408 | RMFm0875RDEvp | 877.262 |
| RMFm0685RDEvp | 166.373 | RMFm0885RDEvp | 413.924 |
| RMFm0696RDEvp | 156.639 | RMFm0896RDEvp | 365.741 |
| RMFm0903RDEvp | 278.159 | RMFm1003RDEvp | 390.516 |
| RMFm0905RDEvp | 206.918 | RMFm1005RDEvp | 293.424 |
| RMFm0975RDEvp | 1100.99 | RMFm1075RDEvp | 1419.93 |
| RMFm0985RDEvp | 701.983 | RMFm1085RDEvp | 894.403 |
| RMFm0996RDEvp | 398.074 | RMFm1096RDEvp | 531.955 |
| RMFm1303RDEvp | 606.263 | RMFm1403RDEvp | 517.695 |
| RMFm1305RDEvp | 461.789 | RMFm1405RDEvp | 402.961 |
| RMFm1375RDEvp | 1830.04 | RMFm1475RDEvp | 1492.51 |
| RMFm1385RDEvp | 1223.78 | RMFm1485RDEvp | 1118.76 |
| RMFm1396RDEvp | 763.753 | RMFm1496RDEvp | 574.25 |
| RMFm1503RDEvp | 1355.35 | RMFm1603RDEvp | 184.983 |
| RMFm1505RDEvp | 1057.95 | RMFm1605RDEvp | 136.964 |
| RMFm1575RDEvp | 3133.29 | RMFm1675RDEvp | 463.796 |
| RMFm1585RDEvp | 2440.82 | RMFm1685RDEvp | 339.403 |
| RMFm1596RDEvp | 1451.72 | RMFm1696RDEvp | 189.96 |

The distribution of multi-family apartments per vintage for SCE’s service territory was derived from Table 2-2 of the June 2000, Final Report on the Statewide Survey of Multi-Family Common Area Building Owners Market, Volume I: Apartment Complexes, by ADM Associates, Inc. and TecMRKT Works LLC.  The distribution is summarized in Table A-1.

| Table A-1. SCE Multi-Family Complexes - Year built |  |  |
| --- | --- | --- |
| Built between 2001 and 2005 | - | - |
| Built 2005 and later - retrofits | - | - |
| Built before 1978 | 6,550 | 49.89% |
| Built between 1978 and 1992 | 5,657 | 43.08% |
| Built between 1993 and 2000 | 553 | 4.21% |
| Subtotal | 12,760 |  |
| Unknown | 370 | 2.82% |
| Grand Total | 13,130 | 100.00% |

A further refinement would breakdown the distribution by each climatic zone, but the detailed data by climate zone was unavailable at the time of this analysis.  Hence, the system-wide distribution was used as representative in each climate zone.  The average size, in square feet, of multi-family dwellings per year built was obtained from the 2003 Residential Appliance Saturation Survey (RASS) database.  For missing vintage data points, older RASS information was used.  Table A-2 through Table A-9 summarizes the values used and the resulting weighted averaged energy consumption for each climate zone.  The Microsoft Excel workbook with the specific calculations is embedded as Reference 3 of this document.  Table A-10 uses the data from Tables A-2 through A-9 to determine the estimated air conditioning consumption for each climate zone.

| Table A-2. Weight Averaging for Climate Zone 6. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RASS | DEER Annual Energy Usage |  | Floor Area (RASS 2003) |  |
| Building Vintage | % of Homes | kWh/1,000sqft | Weighted Average | 1,000 sqft | Weighted Average |
| Built between 2001 and 2005 |  | 147.8 | 0.0 | 1.825 | 0.000 |
| Built 2005 and later - retrofits |  | 106.8 | 0.0 | 1.825 | 0.000 |
| Built before 1978 | 49.9% | 388.4 | 193.8 | 1.027 | 0.512 |
| Built between 1978 and 1992 | 43.1% | 166.4 | 71.7 | 1.210 | 0.521 |
| Built between 1993 and 2000 | 4.2% | 156.6 | 6.6 | 1.293 | 0.054 |
|  | 97.2% |  | 279.9 |  | 1.120 |

| Table A-3. Weight Averaging for Climate Zone 8. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RASS | DEER Annual Energy Usage |  | Floor Area (RASS 2003) |  |
| Building Vintage | % of Homes | kWh/1,000sqft | Weighted Average | 1,000 sqft | Weighted Average |
| Built between 2001 and 2005 |  | 350.9 | 0.0 | 0.819 | 0.000 |
| Built 2005 and later - retrofits |  | 258.1 | 0.0 | 0.819 | 0.000 |
| Built before 1978 | 49.9% | 877.3 | 437.6 | 0.889 | 0.444 |
| Built between 1978 and 1992 | 43.1% | 413.9 | 178.3 | 0.995 | 0.429 |
| Built between 1993 and 2000 | 4.2% | 365.7 | 15.4 | 1.304 | 0.055 |
|  | 97.2% |  | 649.7 |  | 0.954 |

| Table A-4. Weight Averaging for Climate Zone 9. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RASS | DEER Annual Energy Usage |  | Floor Area (RASS 2003) |  |
| Building Vintage | % of Homes | kWh/1,000sqft | Weighted Average | 1,000 sqft | Weighted Average |
| Built between 2001 and 2005 |  | 278.2 | 0.0 | 1.697 | 0.000 |
| Built 2005 and later - retrofits |  | 206.9 | 0.0 | 1.697 | 0.000 |
| Built before 1978 | 49.9% | 1,101.0 | 549.2 | 1.050 | 0.524 |
| Built between 1978 and 1992 | 43.1% | 702.0 | 302.4 | 0.966 | 0.416 |
| Built between 1993 and 2000 | 4.2% | 398.1 | 16.8 | 1.144 | 0.048 |
|  | 97.2% |  | 893.6 |  | 1.017 |

| Table A-5. Weight Averaging for Climate Zone 10. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RASS | DEER Annual Energy Usage |  | Floor Area (RASS 2003) |  |
| Building Vintage | % of Homes | kWh/1,000sqft | Weighted Average | 1,000 sqft | Weighted Average |
| Built between 2001 and 2005 |  | 390.5 | 0.0 | 1.109 | 0.000 |
| Built 2005 and later - retrofits |  | 293.4 | 0.0 | 1.109 | 0.000 |
| Built before 1978 | 49.9% | 1,419.9 | 708.3 | 0.849 | 0.424 |
| Built between 1978 and 1992 | 43.1% | 894.4 | 385.3 | 1.094 | 0.472 |
| Built between 1993 and 2000 | 4.2% | 532.0 | 22.4 | 1.038 | 0.044 |
|  | 97.2% |  | 1,148.5 |  | 0.966 |

| Table A-6. Weight Averaging for Climate Zone 13. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RASS | DEER Annual Energy Usage |  | Floor Area (RASS 2003) |  |
| Building Vintage | % of Homes | kWh/1,000sqft | Weighted Average | 1,000 sqft | Weighted Average |
| Built between 2001 and 2005 |  | 606.3 | 0.0 |  | 0.000 |
| Built 2005 and later - retrofits |  | 461.8 | 0.0 |  | 0.000 |
| Built before 1978 | 49.9% | 1,830.0 | 912.9 | 0.804 | 0.401 |
| Built between 1978 and 1992 | 43.1% | 1,223.8 | 527.3 | 2.259 | 0.973 |
| Built between 1993 and 2000 | 4.2% | 763.8 | 32.2 | 1.010 | 0.043 |
|  | 97.2% |  | 1,515.0 |  | 1.458 |

| Table A-7. Weight Averaging for Climate Zone 14. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RASS | DEER Annual Energy Usage |  | Floor Area (RASS 2003) |  |
| Building Vintage | % of Homes | kWh/1,000sqft | Weighted Average | 1,000 sqft | Weighted Average |
| Built between 2001 and 2005 |  | 517.7 | 0.0 |  | 0.000 |
| Built 2005 and later - retrofits |  | 403.0 | 0.0 |  | 0.000 |
| Built before 1978 | 49.9% | 1,492.5 | 744.5 | 0.902 | 0.450 |
| Built between 1978 and 1992 | 43.1% | 1,118.8 | 482.0 | 1.217 | 0.524 |
| Built between 1993 and 2000 | 4.2% | 574.3 | 24.2 | 0.998 | 0.042 |
|  | 97.2% |  | 1,287.0 |  | 1.046 |

| Table A-8. Weight Averaging for Climate Zone 15. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RASS | DEER Annual Energy Usage |  | Floor Area (RASS 2003) |  |
| Building Vintage | % of Homes | kWh/1,000sqft | Weighted Average | 1,000 sqft | Weighted Average |
| Built between 2001 and 2005 |  | 1,355.4 | 0.0 |  | 0.000 |
| Built 2005 and later - retrofits |  | 1,057.9 | 0.0 |  | 0.000 |
| Built before 1978 | 49.9% | 3,133.3 | 1,563.1 | 1.390 | 0.693 |
| Built between 1978 and 1992 | 43.1% | 2,440.8 | 1,051.6 | 1.290 | 0.556 |
| Built between 1993 and 2000 | 4.2% | 1,451.7 | 61.1 | 0.729 | 0.031 |
|  | 97.2% |  | 2,753.4 |  | 1.317 |

| Table A-9. Weight Averaging for Climate Zone 16. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RASS | DEER Annual Energy Usage |  | Floor Area (RASS 2003) |  |
| Building Vintage | % of Homes | kWh/1,000sqft | Weighted Average | 1,000 sqft | Weighted Average |
| Built between 2001 and 2005 |  | 185.0 | 0.0 |  | 0.000 |
| Built 2005 and later - retrofits |  | 137.0 | 0.0 |  | 0.000 |
| Built before 1978 | 49.9% | 463.8 | 231.4 | 0.949 | 0.473 |
| Built between 1978 and 1992 | 43.1% | 339.4 | 146.2 | 0.725 | 0.312 |
| Built between 1993 and 2000 | 4.2% | 190.0 | 8.0 | 1.456 | 0.061 |
|  | 97.2% |  | 396.8 |  | 0.872 |

| Table A-10. DEER Weighted Average Annual AC Energy Use |  |  |  |
| --- | --- | --- | --- |
| CZ | Weighted Average kWh/1,000sqft | Weighted Avg. Floor Area (1,000 sqft) | Weighted Average Annual kWh |
| 6 | 280 | 1.120 | 313 |
| 8 | 650 | 0.954 | 620 |
| 9 | 894 | 1.017 | 909 |
| 10 | 1,148 | 0.966 | 1,109 |
| 13 | 1,515 | 1.458 | 2,209 |
| 14 | 1,287 | 1.046 | 1,346 |
| 15 | 2,753 | 1.317 | 3,626 |
| 16 | 397 | 0.872 | 346 |